FAQ’s on Multimedia

1) Question:

I have a LCD screen and it has a video composite RCA input and I am using the MSS7100 video switcher. I am using 2 LCD screens in the rear and control station 2 and 3 for them. The TCS101 has a small din plug that will not plug into the LCD screen. How do I make this work?

1) Answer:

You will need to go to page 24 of the MSS7100 owner’s manual. You will see on the top left hand corner the 6-pin layout of the TCS101. You will need to cut open the small din cable and use pin 1 (White) for video signal and pin 2 (Yellow) for video ground and make this a video composite RCA. Then just plug that new RCA right into your LCD. screen.

2) Question:

Using the VRX610 and a DVD player (either DVS9755z or VS715) and I plugged the video output of that unit right into the yellow input of the VRX610 and I am not getting any video at all. What am I doing wrong?

2) Answer:

The way you did it will not work. That input on back of the VRX610 is for the TTX7501z only. You will have to use a CCA-389 cable for the video and audio input on this unit.

3) Question:

How do I use the DVS9755z and the VRX610 together but use only the VRX610 high level speaker output to my speakers?

3) Answer:

You will need a CCA-389 cable for this. Take the Front Audio output of the DVS9755z and the Video output and plug that into the CCA-389 cable. Then plug that into the back of the VRX610. You will also have to change the settings of the DVS9755z so that the Center, Rear, and Sub are all turned off. Do this by pressing the ADJUST button on the DVS9755z and then go to SP SEL and then you can turn these channels off.

4) Question:

I have a VDH910 VCR/TV Tuner and I would like to know which TV Tuner antenna works the best on this unit?

4) Answer:

The ZCB001 is the correct antenna to use on this VCR, because it has the correct F-Type connector. It will screw right in.

5) Question:

Can I use the DVS9755z in a slave mode, so that my VRX610 can control the DVD player and it’s volume level?

5) Answer:

The SLAVE mode on the DVS9755z will only work on the units in Japan. So it will not work here yet. There might be some units that will do this in the upcoming year or two.

6) Question:

On the OHM642 which wireless headphones works with it?

6) Answer:

They are the WH100H or WH100CH headphones. The one with the CH stands for Child size.

7) Question:

The OHM642 has three wires on it for the Dome light. How do you wire this up?

7) Answer:

Well the White wire will always go to the Door Switch. If you have a Negative Trigger Door switch use the Brown wire to 12 volts and the Gray wire to Ground. If you are using a Positive Trigger Door Switch then use the Brown wire to Ground and the Gray wire to 12 volts.

8) Question:

I have the VS715 and I split the video signal with a Y-cable for two LCD screens and now the screens are very dark. How do I fix this?

8) Answer:

You will need to use a Video Amp when using this DVD Player on more than 2 LCD screens. We will have one later this year and the model number will be VA700.

9) Question:

Will the AVC-G1-LT work on a 2001 or 2002 Cadillac Escalade?

9) Answer:

That piece will not work on that center console because it is shorter and wider than the 1999-2000 center consoles.

10) Question:

On the VMA7483 is there a housing or mounting brackets that come with that unit?

10) Answer:

This unit does not have anything like that. You will have to make everything for this unit.